ROBOT OPERATING SYSTEM-ROS2

1. Introduction to ROS 2

What is ROS 2?

ROS 2, or Robot Operating System 2, is the next-generation version of ROS that builds upon the lessons learned from ROS 1 while introducing significant improvements. It is designed to provide a robust, flexible, and scalable framework for building robotic applications. ROS 2 focuses on addressing the limitations of ROS 1, particularly around real-time capabilities, multi-platform support, and scalability.

ROS 2 aims to provide better support for industrial applications, especially for real-time, high-performance, and safety-critical robotic systems. It uses DDS (Data Distribution Service) for communication, allowing for better scalability, fault tolerance, and flexibility.

Key Features of ROS 2:

- Real-Time Communication: Built with real-time constraints in mind, making it suitable for mission-critical robotic applications.

- Cross-platform Support: ROS 2 supports multiple operating systems like Linux, Windows, and macOS, unlike ROS 1 which was Linux-centric.

- Security: Built-in security features like authentication, encryption, and access control.

- Multi-Vendor Middleware: ROS 2 supports DDS middleware, which is flexible and provides better interoperability between different systems and hardware.

- Masterless Architecture: Unlike ROS 1, ROS 2 doesn't require a central master node for communication between nodes.

Overview of ROS (Robot Operating System)

The Robot Operating System (ROS) is a set of software libraries and tools that help developers build robotic applications. It provides a communication framework for building distributed systems, making it easier to interface with hardware, control robot behaviors, simulate robots, and develop higher-level capabilities.

Key Components of ROS:

- Nodes: ROS applications are broken down into nodes. Each node can be a separate process or a thread and is responsible for a specific task, like controlling motors or processing sensor data.

- Topics: Nodes can publish messages to topics, which other nodes can subscribe to in order to receive the data. This allows for communication between different parts of the robotic system.

- Services: In addition to topics, ROS also supports service-based communication, where one node can send a request to another and wait for a response.

- Messages: The data exchanged between nodes is organized into messages, which are defined in .msg files in ROS.

- Actions: Actions provide a way to perform long-running tasks with feedback. This is useful for operations like moving a robot to a target location, where the progress can be monitored in real-time.

Differences Between ROS 1 and ROS 2

ROS 2 brings several significant improvements and changes to overcome the limitations of ROS 1:

| Feature | ROS 1 | ROS 2 |
| --- | --- | --- |
| Middleware | ROS 1 uses custom communication protocol | ROS 2 uses DDS (Data Distribution Service) for communication |
| Real-Time | Limited real-time capabilities | Built-in support for real-time operations |
| Master Node | Central master node required | Masterless architecture (uses DDS for direct peer-to-peer communication) |
| Cross-Platform | Primarily Linux-based | Cross-platform (Linux, Windows, macOS) |
| Security | No built-in security features | Built-in security with DDS (encryption, authentication) |
| Quality of Service (QoS) | Limited QoS features | Enhanced QoS for controlling communication reliability, latency, and bandwidth |
| Lifecycle Management | Limited lifecycle management | Enhanced node lifecycle management for better control of resources |
| Support for Multi-Vendor | Limited interoperability | DDS middleware allows for better integration with multiple vendors and hardware |

ROS 2 Versions

ROS 2 versions are released periodically with new features, fixes, and improvements. Some key versions are:

- ROS 2 Dashing Diademata: The first long-term support (LTS) release in ROS 2, introduced in May 2019.

- ROS 2 Eloquent Elusor: Released in November 2019, with several improvements in stability and features.

- ROS 2 Foxy Fitzroy: The second LTS release, introduced in June 2020. It brought long-term support and improved stability and performance for industrial applications.

- ROS 2 Galactic Geochelone: Released in May 2021, it introduced new features and improvements for ROS 2 in terms of usability and stability.

- ROS 2 Humble Hawksbill: The latest LTS release as of 2022, with new improvements in node management, real-time performance, and better cross-platform support.

- ROS 2 Iron Irwini (Upcoming): Scheduled for release in mid-2023, with further improvements in multi-robot support, real-time capabilities, and tooling.

Each release has a support period (usually 1-2 years), with LTS releases being supported for a longer duration (5 years for Foxy, Humble, and future releases).

ROS 2 Architecture

Nodes

In ROS 2, nodes are the fundamental building blocks of a robotic system. A node is a process that performs a computation or action, such as controlling a motor, reading sensor data, or processing images.

Nodes can:

- Communicate with other nodes via topics, services, or actions.

- Be written in either C++ or Python.

- Be created and managed with the ROS 2 command-line tools (ros2 run, ros2 launch).

Nodes can also have lifecycle management, meaning that a node can be transitioned between different states (e.g., inactive, active, shutting down). This allows for better control over resources in a robotic system.

Topics, Services, and Actions

- Topics: The topic-based communication in ROS 2 allows nodes to send and receive messages asynchronously. A node can publish a message to a topic, and other nodes can subscribe to that topic to receive the messages. Topics are used for continuous or periodic data exchanges, such as sensor data or status updates.

- Services: Services allow synchronous communication between nodes. A service server waits for requests, processes the request, and sends back a response. A service client can make a request and wait for the server’s response. Services are ideal for tasks that require a request/response pattern, such as setting a robot's position or controlling a specific robot feature.

- Actions: Actions are similar to services, but they are designed for long-running tasks. Actions allow clients to send a request and receive feedback during the operation, along with the final result once completed. Actions are useful for tasks like moving a robot to a destination, where you want to monitor the progress and possibly cancel or modify the action.

ROS 2 Communication Model (DDS)

ROS 2 adopts the Data Distribution Service (DDS) as its underlying communication framework. DDS is an open standard for real-time, publish-subscribe communication used in many industries for mission-critical applications. DDS provides several key advantages over the traditional ROS 1 communication model:

- Peer-to-Peer Communication: DDS allows direct communication between nodes, eliminating the need for a central master node like in ROS 1. This increases scalability and reliability, especially in large robotic systems.

- Quality of Service (QoS): DDS offers flexible QoS policies that allow you to configure parameters like reliability, durability, and latency of communication, which is critical for real-time systems.

- Security: DDS includes built-in security mechanisms like authentication, encryption, and access control, providing a more secure communication model for robotic applications.

- Scalability and Fault Tolerance: DDS handles complex, distributed systems with ease and ensures robustness in large-scale or fault-tolerant systems.

Masterless System

One of the key differences in ROS 2 compared to ROS 1 is the masterless architecture. In ROS 1, the system relies on a central ROS Master to coordinate communication between nodes. The master is responsible for managing node registration and topic connections.

In contrast, ROS 2 uses DDS, which allows for peer-to-peer communication between nodes without the need for a central master. This not only simplifies the architecture but also improves scalability, flexibility, and robustness, as nodes can directly discover and communicate with each other without depending on a single point of failure.

ROS 2 Middleware (DDS, RTPS)

ROS 2's communication is powered by DDS (Data Distribution Service), a middleware standard used for real-time, reliable, and scalable communication. DDS offers several key features:

- RTPS (Real-Time Publish-Subscribe): ROS 2 uses the RTPS protocol, a part of DDS, for real-time communication. RTPS enables communication between nodes with low-latency and high-reliability, which is essential for real-time applications like robotics.

- Customizable QoS: DDS allows for defining Quality of Service (QoS) settings that control message delivery policies, such as reliability, durability, and message size.

- Flexible Data Exchange: DDS supports various data exchange models, including point-to-point communication and multicast, providing flexibility for different types of robotic systems.

ROS 2 Workspace Creation

A ROS 2 workspace is a directory where ROS 2 packages are organized, built, and managed.
It typically contains:

<workspace>/

├── src/         # Source folder for ROS 2 packages

├── build/       # Build system files (created after build)

├── install/     # Installed products (created after build)

└── log/         # Logs from build process

Steps to Create a ROS 2 Workspace

1. Create the Workspace Directory

Create a new workspace directory with a src subfolder:

mkdir -p ~/ros2_ws/src

- ~/ros2_ws is the name of the workspace (can be renamed as needed, for example my_robot_ws).

- src is the subfolder where all ROS 2 packages will be placed.

2. Navigate to the Workspace

Move into the workspace directory:

cd ~/ros2_ws

3. Build the Workspace

Use colcon, the ROS 2 build tool, to build the workspace:

colcon build

This command:

- Searches the src/ directory for ROS 2 packages.

- Compiles them.

- Creates the build/, install/, and log/ directories.

4. Source the Workspace

After building, you need to source the environment to use the workspace:

source install/setup.bash

Notes:

- This step must be repeated in each terminal where you want to use the workspace.

- To make sourcing automatic, add the following to your .bashrc:

echo "source ~/ros2_ws/install/setup.bash" >> ~/.bashrc

ROS 2 Package Creation

In ROS 2, a package is the fundamental unit of organization, containing nodes, libraries, configuration files, launch files, and other resources. All ROS 2 development is done inside packages.

Steps to Create a ROS 2 Package

1. Navigate to the src Directory

Go to the src folder of your ROS 2 workspace:

cd ~/ros2_ws/src

This ensures the package will be placed inside the workspace.

2. Create the Package

Use the ros2 pkg create command.

For a Python package:

ros2 pkg create --build-type ament_python <package_name>

For a C++ package:

ros2 pkg create --build-type ament_cmake <package_name>

Example (Python):

ros2 pkg create --build-type ament_python my_robot_pkg

Example (C++):

ros2 pkg create --build-type ament_cmake my_cpp_robot_pkg

Common Options

- --dependencies → List of dependent packages (separated by space)

Example:

ros2 pkg create --build-type ament_python my_robot_pkg --dependencies rclpy std_msgs

- --license → Specify a license

Example:

ros2 pkg create --build-type ament_cmake my_cpp_pkg --license Apache-2.0

3. Build the Package

Return to the root of the workspace:

cd ~/ros2_ws

Then build:

colcon build

4. Source the Workspace

After building, source the workspace:

source install/setup.bash

5. Verify the Package

List available packages to check if your new package is detected:

ros2 pkg list | grep <package_name>

Example:

ros2 pkg list | grep my_robot_pkg

Generated Package Structure

For a Python package (ament_python), the structure typically looks like:

my_robot_pkg/

├── package.xml

├── resource/

├── setup.cfg

├── setup.py

├── test/

└── my_robot_pkg/

└── __init__.py

For a C++ package (ament_cmake), it typically looks like:

my_cpp_robot_pkg/

├── CMakeLists.txt

├── package.xml

└── src/

Important Files

- package.xml → Defines package metadata, dependencies, and build info.

- CMakeLists.txt → Build configuration file (for C++ packages).

- setup.py and setup.cfg → Python packaging files (for Python packages).

Example Commands

- Create Python package with dependencies:

ros2 pkg create --build-type ament_python my_robot_pkg --dependencies rclpy std_msgs

- Create C++ package with dependencies:

ros2 pkg create --build-type ament_cmake my_cpp_robot_pkg --dependencies rclcpp std_msgs

Adding Nodes or Code

After package creation:

- Python nodes go inside the <package_name>/ directory.

- C++ source files go inside src/ and are referenced in CMakeLists.txt.

Do not forget to update:

- package.xml with any additional dependencies.

- setup.py (Python) or CMakeLists.txt (C++) when adding executables or libraries.

Troubleshooting

- If package does not appear after colcon build, check:

  - You are inside the correct workspace.

  - You have sourced the install/setup.bash.

  - There are no syntax or configuration errors in package.xml or setup.py/CMakeLists.txt.

If you want, I can also prepare ready-to-use code templates or package skeletons for common use cases (e.g., a publisher-subscriber package, service-action package). Let me know if you’d like those.

ROS 2 Node Creation

In ROS 2, a node is an executable that communicates with other nodes using topics, services, or actions. Nodes are organized inside ROS 2 packages.

Below are the steps for creating nodes in both Python and C++.

Python Node Creation

1. Prerequisites

- ROS 2 workspace and package already created (with ament_python).

- Dependencies such as rclpy declared in package.xml and setup.py.

2. Create Python Node File

Inside your package directory:

cd ~/ros2_ws/src/my_robot_pkg/my_robot_pkg/

Create a new Python script, for example:

touch simple_node.py

chmod +x simple_node.py

3. Example Python Node Code

File: simple_node.py

#!/usr/bin/env python3

import rclpy

from rclpy.node import Node

class SimpleNode(Node):

def __init__(self):

super().__init__('simple_node')

self.get_logger().info('SimpleNode has been started')

def main(args=None):

rclpy.init(args=args)

node = SimpleNode()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

4. Register Executable in setup.py.

Edit the setup.py file in your package:

entry_points={

'console_scripts': [

'simple_node = my_robot_pkg.simple_node:main',

],

},

This tells ROS 2 how to launch the node.

5. Build the Package

From the workspace root:

cd ~/ros2_ws

colcon build

6. Source the Workspace

source install/setup.bash

7. Run the Node

ros2 run my_robot_pkg simple_node

C++ Node Creation

1. Prerequisites

- ROS 2 workspace and package already created (with ament_cmake).

- Dependencies such as rclcpp declared in package.xml and CMakeLists.txt.

2. Create C++ Source File

Inside your package:

cd ~/ros2_ws/src/my_cpp_robot_pkg/src/

touch simple_node.cpp

3. Example C++ Node Code

File: simple_node.cpp

#include "rclcpp/rclcpp.hpp"

class SimpleNode : public rclcpp::Node

{

public:

SimpleNode() : Node("simple_node")

{

RCLCPP_INFO(this->get_logger(), "SimpleNode has been started");

}

};

int main(int argc, char * argv[])

{

rclcpp::init(argc, argv);

rclcpp::spin(std::make_shared<SimpleNode>());

rclcpp::shutdown();

return 0;

}

4. Update CMakeLists.txt

- Add the executable and link libraries:

add_executable(simple_node src/simple_node.cpp)

ament_target_dependencies(simple_node rclcpp)

install(TARGETS

simple_node

DESTINATION lib/${PROJECT_NAME})

5. Build the Package

From the workspace root:

cd ~/ros2_ws

colcon build

6. Source the Workspace

source install/setup.bash

7. Run the Node

ros2 run my_cpp_robot_pkg simple_node

ROS 2 Simple Node

We will create:

- Package: my_robot_pkg

- Python node: simple_node.py

1. Navigate to Workspace

cd ~/ros2_ws/src

2. Create the Package

ros2 pkg create --build-type ament_python my_robot_pkg --dependencies rclpy

This creates the package folder my_robot_pkg/ with necessary files.

3. Write the Node Script

Navigate to the package’s internal directory:

cd ~/ros2_ws/src/my_robot_pkg/my_robot_pkg

Create the Python node file:

touch simple_node.py

chmod +x simple_node.py

4. Add Node Code

Edit simple_node.py and add:

#!/usr/bin/env python3

import rclpy

from rclpy.node import Node

class SimpleNode(Node):

def __init__(self):

super().__init__('simple_node')

self.get_logger().info('SimpleNode has been started')

def main(args=None):

rclpy.init(args=args)

node = SimpleNode()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

5. Update setup.py

Go to the package root:

cd ~/ros2_ws/src/my_robot_pkg

Edit setup.py and make sure you have this under entry_points:

entry_points={

'console_scripts': [

'node_simple = my_robot_pkg.simple_node:main',

],

},

This tells ROS 2 how to run the script as a command.

6. Build the Workspace

From the workspace root:

cd ~/ros2_ws

colcon build

7. Source the Setup

source install/setup.bash

8. Run the Node

ros2 run my_robot_pkg node_simple

Expected terminal output:

[INFO] [<timestamp>] [simple_node]: SimpleNode has been started

Folder Structure After Setup

ros2_ws/

├── src/

│    └── my_robot_pkg/

│         ├── my_robot_pkg/

│         │    └── simple_node.py

│         ├── package.xml

│         ├── setup.py

│         ├── setup.cfg

│         └── resource/

├── build/

├── install/

└── log/

Continuous ROS 2 Python Node

Navigate to Package Directory

cd ~/ros2_ws/src/my_robot_pkg/my_robot_pkg

touch continuous_node.py

chmod +x continuous_node.py

#!/usr/bin/env python3

import rclpy

from rclpy.node import Node

class ContinuousNode(Node):

def __init__(self):

super().__init__('continuous_node')

# Set timer: 1.0 second (1000 ms)

self.timer = self.create_timer(1.0, self.timer_callback)

self.get_logger().info('ContinuousNode has been started')

def timer_callback(self):

self.get_logger().info('ContinuousNode is running...')

def main(args=None):

rclpy.init(args=args)

node = ContinuousNode()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

Update setup.py

Edit setup.py and make sure you have this under entry_points:

entry_points={

'console_scripts': [

'simple_node = my_robot_pkg.simple_node:main',

'continuous_node = my_robot_pkg.continuous_node:main',

],

},

Run the Continuous Node

ros2 run my_robot_pkg continuous_node

ROS COMMUNICATION

ROS 2 Publisher and Subscriber – Step-by-Step (Python)

Step 1: Create a ROS 2 Python Package

cd ~/ros2_ws/src

ros2 pkg create --build-type ament_python simple_pubsub --dependencies rclpy std_msgs

This creates a folder simple_pubsub with necessary files.

Step 2: Write the Publisher Node

Create the file simple_publisher.py inside the package:

cd ~/ros2_ws/src/simple_pubsub/simple_pubsub

gedit simple_publisher.py

simple_publisher.py:

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class SimplePublisher(Node):

def __init__(self):

super().__init__('simple_publisher')

self.publisher_ = self.create_publisher(String, 'topic', 10)

timer_period = 1  # seconds

self.timer = self.create_timer(timer_period, self.publish_message)

self.count = 0

def publish_message(self):

msg = String()

msg.data = f'Hello ROS 2! Count: {self.count}'

self.publisher_.publish(msg)

self.get_logger().info(f'Published: "{msg.data}"')

self.count += 1

def main(args=None):

rclpy.init(args=args)

node = SimplePublisher()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

Step 3: Write the Subscriber Node

Create the file simple_subscriber.py:

gedit simple_subscriber.py

simple_subscriber.py:

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class SimpleSubscriber(Node):

def __init__(self):

super().__init__('simple_subscriber')

self.subscription = self.create_subscription(

String,

'topic',

self.listener_callback,

10

)

def listener_callback(self, msg):

self.get_logger().info(f'Received: "{msg.data}"')

def main(args=None):

rclpy.init(args=args)

node = SimpleSubscriber()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

Step 4: Update setup.py

Open setup.py and add both nodes under entry_points:

entry_points={

'console_scripts': [

'simple_publisher = simple_pubsub.simple_publisher:main',

'simple_subscriber = simple_pubsub.simple_subscriber:main',

],

},

Step 5: Make Python Files Executable

chmod +x simple_publisher.py

chmod +x simple_subscriber.py

Step 6: Build the Package

Go to your workspace root and build:

cd ~/ros2_ws

colcon build

Step 7: Source the Workspace

source install/setup.bash

Step 8: Run the Nodes

In two terminals:

Terminal 1 – Run Publisher

ros2 run simple_pubsub simple_publisher

Terminal 2 – Run Subscriber

ros2 run simple_pubsub simple_subscriber

You will see:

[INFO] [publisher]: Published: "Hello ROS 2! Count: 0"

[INFO] [subscriber]: Received: "Hello ROS 2! Count: 0"

...

ROS 2 Custom Service: Add Three Integers (Python)

Step 1: Create a Service Interface Package (ament_cmake)

ros2 pkg create --build-type ament_cmake add_three_ints_interfaces

cd add_three_ints_interfaces

mkdir srv

Create the Service Definition File

Create a file named:

srv/AddThreeInts.srv

Paste this content:

int64 a

int64 b

int64 c

---

int64 sum

Edit package.xml

Ensure these lines exist:

<buildtool_depend>ament_cmake</buildtool_depend>

<build_depend>rosidl_default_generators</build_depend>

<exec_depend>rosidl_default_runtime</exec_depend>

<member_of_group>rosidl_interface_packages</member_of_group>

Edit CMakeLists.txt

Replace contents with:

find_package(ament_cmake REQUIRED)

find_package(rosidl_default_generators REQUIRED)

rosidl_generate_interfaces(${PROJECT_NAME}

"srv/AddThreeInts.srv"

)

Build the Interface Package

cd ~/your_workspace

colcon build --packages-select add_three_ints_interfaces

source install/setup.bash

Step 2: Create Python Package for Server and Client

ros2 pkg create --build-type ament_python add_three_ints_py

cd add_three_ints_py

mkdir add_three_ints_py

touch add_three_ints_py/__init__.py

Edit package.xml

Add these dependencies:

<exec_depend>rclpy</exec_depend>

<exec_depend>add_three_ints_interfaces</exec_depend>

Edit setup.py

Update the file to look like this:

from setuptools import setup

package_name = 'add_three_ints_py'

setup(

name=package_name,

version='0.0.0',

packages=[package_name],

install_requires=['setuptools'],

zip_safe=True,

maintainer='Your Name',

maintainer_email='your_email@example.com',

description='Python server and client for AddThreeInts service',

license='Apache License 2.0',

tests_require=['pytest'],

entry_points={

'console_scripts': [

'add_three_ints_server = add_three_ints_py.add_three_ints_server:main',

'add_three_ints_client = add_three_ints_py.add_three_ints_client:main',

],

},

)

Step 3: Create the Service Server Node

Create file:

add_three_ints_py/add_three_ints_server.py

Paste this code:

import rclpy

from rclpy.node import Node

from add_three_ints_interfaces.srv import AddThreeInts

class AddThreeIntsServer(Node):

def __init__(self):

super().__init__('add_three_ints_server')

self.srv = self.create_service(AddThreeInts, 'add_three_ints', self.callback)

self.get_logger().info('Service server ready.')

def callback(self, request, response):

response.sum = request.a + request.b + request.c

self.get_logger().info(f"Request: {request.a} + {request.b} + {request.c} = {response.sum}")

return response

def main(args=None):

rclpy.init(args=args)

node = AddThreeIntsServer()

rclpy.spin(node)

rclpy.shutdown()

if __name__ == '__main__':

main()

OR

import rclpy

from rclpy.node import Node

from add_three_ints_interfaces.srv import AddThreeInts

class Server(Node):

def __init__(self):

super().__init__('server')

self.create_service(AddThreeInts, 'add_three_ints', self.callback)

self.get_logger().info('Server is ready.')

def callback(self, request, response):

response.sum = request.a + request.b + request.c

return response

def main():

rclpy.init()

rclpy.spin(Server())

rclpy.shutdown()

Step 4: Create the Service Client Node

Create file:

add_three_ints_py/add_three_ints_client.py

Paste this code:

import sys

import rclpy

from rclpy.node import Node

from add_three_ints_interfaces.srv import AddThreeInts

class AddThreeIntsClient(Node):

def __init__(self):

super().__init__('add_three_ints_client')

self.cli = self.create_client(AddThreeInts, 'add_three_ints')

while not self.cli.wait_for_service(timeout_sec=1.0):

self.get_logger().info('Waiting for service...')

self.req = AddThreeInts.Request()

def send_request(self, a, b, c):

self.req.a = a

self.req.b = b

self.req.c = c

self.future = self.cli.call_async(self.req)

def main(args=None):

rclpy.init(args=args)

node = AddThreeIntsClient()

if len(sys.argv) != 4:

node.get_logger().info('Usage: add_three_ints_client a b c')

return

a, b, c = map(int, sys.argv[1:4])

node.send_request(a, b, c)

while rclpy.ok():

rclpy.spin_once(node)

if node.future.done():

try:

response = node.future.result()

except Exception as e:

node.get_logger().error(f'Service call failed: {e}')

else:

node.get_logger().info(f'Result: {response.sum}')

break

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

OR

import rclpy

from rclpy.node import Node

from add_three_ints_interfaces.srv import AddThreeInts

import sys

class Client(Node):

def __init__(self):

super().__init__('client')

self.cli = self.create_client(AddThreeInts, 'add_three_ints')

while not self.cli.wait_for_service(timeout_sec=1.0):

self.get_logger().info('Waiting for service...')

self.request = AddThreeInts.Request()

def send(self, a, b, c):

self.request.a = a

self.request.b = b

self.request.c = c

future = self.cli.call_async(self.request)

rclpy.spin_until_future_complete(self, future)

if future.result():

self.get_logger().info(f'Result: {future.result().sum}')

else:

self.get_logger().error('Service call failed.')

def main():

rclpy.init()

client = Client()

a, b, c = map(int, sys.argv[1:4])

client.send(a, b, c)

rclpy.shutdown()

Step 5: Build and Run

Build:

cd ~/your_workspace

colcon build --packages-select add_three_ints_py

source install/setup.bash

Run the server:

ros2 run add_three_ints_py add_three_ints_server

Run the client:

In a new terminal:

source install/setup.bash

ros2 run add_three_ints_py add_three_ints_client 5 10 15

Expected output: Result: 30

ROS2 Custom Message

Creating and Using a Custom ROS 2 Message (Python)

This document describes how to define, build, and use a custom message type in a ROS 2 Python package.

Step 1: Create a New ROS 2 Package (ament_cmake)

Open a terminal and run the following command:

cd ~/ros2_ws/src

ros2 pkg create custom_interfaces --build-type ament_cmake --dependencies std_msgs

Step 2: Create the msg Directory and Custom Message File

Navigate into the package and create the msg directory:

cd ~/ros2_ws/src/custom_interfaces

mkdir msg

cd msg

touch Person.msg

Open Person.msg and define the custom message format:

string name

int32 age

This message consists of a string name and an integer age.

Step 3: Modify CMakeLists.txt

Open CMakeLists.txt and add the following lines:

cmake_minimum_required(VERSION 3.5)

project(custom_interfaces)

find_package(ament_cmake REQUIRED)

find_package(std_msgs REQUIRED)

find_package(rosidl_default_generators REQUIRED)

rosidl_generate_interfaces(${PROJECT_NAME}

"msg/Person.msg"

DEPENDENCIES std_msgs

)

ament_export_dependencies(rosidl_default_runtime)

ament_package()

Step 4: Modify package.xml

Add the following dependencies inside <build_depend> and <exec_depend> tags:

<buildtool_depend>ament_cmake</buildtool_depend>

<build_depend>rosidl_default_generators</build_depend>

<exec_depend>rosidl_default_runtime</exec_depend>

<depend>std_msgs</depend>

<export>

<build_type>ament_cmake</build_type>

</export>

Step 5: Modify setup.py

Edit setup.py as follows:

from setuptools import setup

import os

from glob import glob

package_name = 'custom_msg_pkg'

setup(

name=package_name,

version='0.0.0',

packages=[package_name],

data_files=[

('share/' + package_name, ['package.xml']),

(os.path.join('share', package_name, 'msg'), glob('msg/*.msg')),

],

install_requires=['setuptools'],

zip_safe=True,

maintainer='your_name',

maintainer_email='your_email@example.com',

description='Package with a custom message',

license='MIT',

tests_require=['pytest'],

entry_points={

'console_scripts': [

'person_publisher = custom_msg_pkg.person_publisher:main',

'person_subscriber = custom_msg_pkg.person_subscriber:main',

],

},

)

Step 6: Create Python Package Folder

cd ~/ros2_ws/src/custom_msg_pkg

mkdir custom_msg_pkg

cd   custom_msg_pkg

Step 7: Write the Publisher Node

Create the file:

cd ~/ros2_ws/src/custom_msg_pkg/custom_msg_pkg

gedit person_publisher.py

Add the following code:

import rclpy

from rclpy.node import Node

from custom_msg_pkg.msg import Person

class PersonPublisher(Node):

def __init__(self):

super().__init__('person_publisher')

self.publisher_ = self.create_publisher(Person, 'person_info', 10)

self.timer = self.create_timer(2.0, self.publish_person)

self.count = 0

def publish_person(self):

msg = Person()

msg.name = 'Alice'

msg.age = 25 + self.count

self.publisher_.publish(msg)

self.get_logger().info(f'Published: name="{msg.name}", age={msg.age}')

self.count += 1

def main(args=None):

rclpy.init(args=args)

node = PersonPublisher()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

Step 8: Write the Subscriber Node

Create the file:

gedit person_subscriber.py

Add the following code:

import rclpy

from rclpy.node import Node

from custom_msg_pkg.msg import Person

class PersonSubscriber(Node):

def __init__(self):

super().__init__('person_subscriber')

self.subscription = self.create_subscription(

Person,

'person_info',

self.callback,

10

)

def callback(self, msg):

self.get_logger().info(f'Received: name="{msg.name}", age={msg.age}')

def main(args=None):

rclpy.init(args=args)

node = PersonSubscriber()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

Step 9: Make Python Files Executable

chmod +x person_publisher.py

chmod +x person_subscriber.py

Step 10: Build the Package

Return to the root of the workspace and build:

cd ~/ros2_ws

colcon build --packages-select custom_msg_pkg

Step 11: Source the Setup File

source install/setup.bash

Step 12: Run the Publisher and Subscriber

Use two terminals:

Terminal 1 – Publisher:

ros2 run custom_msg_pkg person_publisher

Terminal 2 – Subscriber:

ros2 run custom_msg_pkg person_subscriber

Output Example

Publisher:

Published: name="Alice", age=25

Published: name="Alice", age=26

...

Subscriber:

Received: name="Alice", age=25

Received: name="Alice", age=26

...